EE 533 Lab 9 — Architecture Interface Specification

Network Processor with ARM CPU and GPU Tensor Core

Team 10 (CPU) + Team 2 (GPU)  |  EE533 Spring 2026

Shared Design Document for CPU Subgroup and GPU Subgroup

1. System Overview

This document defines the architecture and interface specifications for the integrated network processor system on NetFPGA. The system is developed by two subgroups: the CPU subgroup (Team 10) provides the ARM processor, Convertible FIFO, and system integration framework; the GPU subgroup (Team 2) provides the BFloat16 tensor processing unit.

Both subgroups' modules connect to a shared Block RAM (convertible_fifo) through a common interface defined in this document. The system sits within the NetFPGA user_data_path module, capturing network packets into shared BRAM and processing them with the ARM CPU and/or GPU.

2. Top-Level Architecture

2.1 Module Hierarchy

user_data_path.v

+-- fifo_top.v                  (system integration + register I/F)

+-- convertible_fifo.v     (256x72 shared BRAM, packet I/O)

|     Port A: network capture/send

|     Port B: MUX -> host / ARM / GPU

|

+-- arm_cpu_wrapper.v      [CPU SUBGROUP - Team 10]

|     +-- arm_pipeline.v   (4-thread ARM pipeline IF/ID/EX/MEM/WB)

|     +-- arm_alu.v        (32-bit ALU)

|     +-- arm_regfile.v    (dual mirrored BRAM, 64x32)

|     +-- imem.v           (instruction ROM, async case)

|     +-- dmem_32.v        (256x32 data memory)

|

+-- gpu_net.v              [GPU SUBGROUP - Team 2]

+-- gpu_core_min.v   (programmable GPU: 7-state multi-cycle FSM)

+-- gpu_imem_rom.v   (GPU instruction ROM: bf16_fma kernel)

+-- gpu_regfile_min.v(8x64 BRAM register file)

+-- gpu_ldst.v       (LD64/ST64 load-store unit)

+-- tensor_unit.v    (4-lane parallel BF16 MAC)

+-- bf16_mac.v       (BF16 fused multiply-accumulate)

+-- isa_defines.vh   (GPU ISA opcode definitions)

2.2 Responsibility Boundary

| CPU Subgroup Provides (Team 10) | GPU Subgroup Provides (Team 2) |
| --- | --- |
| fifo_top.v (top-level wrapper) | gpu_net.v + gpu_core_min.v |
| convertible_fifo.v (shared BRAM) | gpu_imem_rom.v, gpu_regfile_min.v |
| arm_cpu_wrapper.v + ARM pipeline | gpu_ldst.v, tensor_unit.v, bf16_mac.v |
| user_data_path.v (NetFPGA integration) | isa_defines.vh |
| Register interface + control scripts | kernel.cu, kernel.ptx (CUDA toolchain) |
| Port B MUX logic | ptx_parser.py, bf16_utils.py |

3. GPU Module Interface (gpu_net.v)

This is the critical interface that both subgroups must agree on. The GPU module implements the following port list to connect to fifo_top.v:

3.1 Port Definition

| Direction | Signal | Width | Description |
| --- | --- | --- | --- |
| input | clk | 1 | System clock (125 MHz NetFPGA clock) |
| input | rst | 1 | Active-high synchronous reset (converted from rst_n by gpu_net adapter) |
| input | start | 1 | Level signal: high while GPU should run; edge-detected internally |
| output | done | 1 | High when execution complete; stays high until next start |
| output | running | 1 | High while GPU is actively executing |
| output | bram_addr | 8 | BRAM word address for read or write (0-255) |
| output | bram_wdata | 72 | Write data: {ctrl[7:0], data[63:0]} |
| input | bram_rdata | 72 | Read data: {ctrl[7:0], data[63:0]} (1-cycle latency) |
| output | bram_we | 1 | Write enable (active high, single-cycle pulse) |

Reset polarity change: The initial spec used rst_n (active-low). The implemented gpu_net.v adapter converts rst_n from fifo_top to active-high rst for gpu_core_min, eliminating a mismatch bug.

3.2 Verilog Module Signature

module gpu_net (

input  wire        clk,

input  wire        rst_n,      // active-low from fifo_top

input  wire        start,      // level signal from fifo_top

output wire        done,

output wire        running,

output wire [7:0]  bram_addr,

output wire [71:0] bram_wdata,

input  wire [71:0] bram_rdata,

output wire        bram_we

);

// gpu_net internally inverts rst_n -> rst for gpu_core_min

wire rst = ~rst_n;

3.3 BRAM Timing

The BRAM (convertible_fifo) uses synchronous reads with 1-cycle latency. The GPU FSM uses two states (S_MEM + S_MEM2) to handle this correctly:

| Cycle | FSM State | Action |
| --- | --- | --- |
| T | S_EXEC | GPU computes bram_addr, stores in register |
| T+1 | S_MEM | bram_addr presented to BRAM port B input |
| T+2 | S_MEM2 | bram_rdata is valid — GPU captures data into exec_result |

3.4 Control Protocol

The start signal is generated by fifo_top when the host writes CMD register with bit[4]=1. The GPU edge-detects start internally:

- On rising edge of start: assert running=1, deassert done, reset PC to 0, enter S_FETCH

- During execution: running=1, done=0. Access BRAM freely via bram_addr/bram_we

- On completion (PC reaches last instruction + WB): assert done=1, deassert running=0, enter S_DONE

- done stays high until the next start pulse, allowing host to poll PROC_STATUS[2]

4. Shared BRAM Memory Map

The convertible_fifo BRAM is 256 entries x 72 bits. Each entry stores {ctrl[7:0], data[63:0]}. The following memory map reflects the final implemented design:

| BRAM Addr | Owner | Contents |
| --- | --- | --- |
| 0 | GPU I/O | Vec A input (4 packed BF16, 64-bit) + GPU result output after execution |
| 1 | GPU in | Vec B input: 4 packed BF16 values (e.g. 1.5 x4 = 0x3FC03FC0_3FC03FC0) |
| 2 | GPU in | Vec C input: 4 packed BF16 values (e.g. 0.5 x4 = 0x3F003F00_3F003F00) |
| 3-5 | Reserved | Available; not used in current kernel |
| 6 | ARM I/O | ARM header word 0: {hi[31:0], lo[31:0]} — ARM adds 1 to each 32-bit field |
| 7 | ARM I/O | ARM header word 1: {hi[31:0], lo[31:0]} — ARM adds 1 to each 32-bit field |
| 8-12 | ARM I/O | ARM data words 2-6 (dmem words, adds 1 per field) |
| 13-255 | Reserved | Packet payload / future use |

4.1 BFloat16 Packing Convention

Each 64-bit data word contains 4 packed BFloat16 values:

data[63:48] = lane 3 (MSB)

data[47:32] = lane 2

data[31:16] = lane 1

data[15:0]  = lane 0 (LSB)

Example: 2.0 in BF16 = 0x4000. Packed x4 across 64 bits:

0x4000_4000_4000_4000  →  all 4 lanes = 2.0

5. Port B Access Priority

BRAM Port B is shared between three consumers. fifo_top.v implements a priority MUX:

| Priority | Consumer | Condition |
| --- | --- | --- |
| 1 (highest) | ARM CPU | arm_running == 1 |
| 2 | GPU | gpu_running == 1 |
| 3 (lowest) | Host regwrite | Neither ARM nor GPU running |

ARM and GPU must NOT run simultaneously. The host controls execution order via the CMD register. Typical sequence: write data → run ARM → run GPU → read results.

6. Host Register Interface

The host controls the system via software-written registers accessed through regwrite/regread (fifoctl). All register logic is handled by the CPU subgroup in fifo_top.v.

6.1 CMD Register (0x2000100) — Software Writable

| Bits | Field | Description |
| --- | --- | --- |
| [1:0] | mode | 00=IDLE (passthrough), 01=PROC, 10=FIFO_OUT, 11=FIFO_IN |
| [2] | reset | Write 1 to reset FIFO pointers |
| [3] | arm_start | Rising edge triggers ARM execution (edge-detected in fifo_top) |
| [4] | gpu_start | Rising edge triggers GPU execution (edge-detected in fifo_top) |

6.2 PROC_STATUS Register (0x2000124) — Hardware Readable

| Bits | Field | Description |
| --- | --- | --- |
| [0] | arm_done | 1 when ARM execution is complete |
| [1] | arm_running | 1 while ARM is executing |
| [2] | gpu_done | 1 when GPU execution is complete |
| [3] | gpu_running | 1 while GPU is executing |

7. GPU ISA Summary

The GPU implements a custom 32-bit RISC ISA defined in isa_defines.vh. Instructions are 32 bits wide with 6-bit opcode:

| Opcode (6b) | Mnemonic | Format | Operation |
| --- | --- | --- | --- |
| 000001 | READ_TID | R-type | Rd = threadIdx.x (hardwired 0) |
| 000011 | LD64 | I-type | Rd = BRAM[Rs_base + imm*1] (64-bit load) |
| 000100 | ST64 | I-type | BRAM[Rs_base + imm] = Rd (64-bit store) |
| 010000 | ADD_I16 | R-type | Rd = Rs1 + Rs2 (16-bit integer) |
| 100001 | BF_MAC | R-type | Rd = Rs1*Rs2 + Rs3 (BF16 FMA, 4 lanes) |
| 100010 | RELU | R-type | Rd = max(0, Rs1) per lane |

7.1 Instruction Encoding

R-type: [31:26]=opcode [25:22]=Rd [21:18]=Rs1 [17:14]=Rs2 [13:10]=Rs3 [9:0]=unused

I-type: [31:26]=opcode [25:22]=Rd [21:18]=Rs_base [17:0]=imm18

8. CUDA Software Stack

The GPU subgroup (Team 2) provides a complete software toolchain demonstrating the CUDA-to-hardware mapping. The toolchain translates a real CUDA kernel through PTX assembly into GPU ISA machine code.

8.1 Toolchain Flow

kernel.cu                          # CUDA source (nvcc-compilable)

|

v  nvcc --ptx (NVIDIA CUDA 13.1)

kernel.ptx                         # Real PTX intermediate representation

|

v  python ptx_parser.py kernel.ptx

gpu_program.hex                    # GPU ISA machine code (6 instructions)

8.2 bf16_fma CUDA Kernel

// kernel.cu

__global__ void bf16_fma(__nv_bfloat16 *a, __nv_bfloat16 *b,

__nv_bfloat16 *c, __nv_bfloat16 *d) {

int idx = threadIdx.x;

d[idx] = __hfma(a[idx], b[idx], c[idx]);  // d = a*b + c

}

8.3 PTX → GPU ISA Mapping

ptx_parser.py compiles the bf16_fma kernel from PTX to GPU ISA. The generated output matches gpu_imem_rom.v exactly:

| PC | PTX Instruction | GPU ISA | Operation |
| --- | --- | --- | --- |
| 0 | (injected by parser) | READ_TID R1 | R1 = threadIdx.x = 0 |
| 1 | ld.global.u16 %rs2, [%rd10] | LD64 R2, R0, 0 | R2 = BRAM[0] (Vec A) |
| 2 | ld.global.u16 %rs3, [%rd11] | LD64 R3, R0, 8 | R3 = BRAM[1] (Vec B) |
| 3 | ld.global.u16 %rs4, [%rd12] | LD64 R4, R0, 16 | R4 = BRAM[2] (Vec C) |
| 4 | fma.rn.bf16 %rs1,%rs2,%rs3,%rs4 | BF_MAC R5, R2, R3, R4 | R5 = R2*R3+R4 |
| 5 | st.global.u16 [%rd13], %rs1 | ST64 R5, R1, 0 | BRAM[0] = R5 (result) |

8.4 BFloat16 Utility (bf16_utils.py)

bf16_utils.py provides Python-level BF16 conversion for test vector generation and result verification:

from bf16_utils import float32_to_bfloat16, bfloat16_to_float32

# Generate test vectors

a = float32_to_bfloat16(2.0)   # 0x4000

b = float32_to_bfloat16(1.5)   # 0x3FC0

c = float32_to_bfloat16(0.5)   # 0x3F00

# Verify result

result = bfloat16_to_float32(0x4060)  # 3.5

9. System Execution Flow

Complete execution flow for the bf16_fma computation on hardware:

| Step | Host Action | Hardware Effect |
| --- | --- | --- |
| 1 | Write CMD=0x04 (reset), then 0x00 | FIFO pointers reset, system idle |
| 2 | Write CMD=0x01 (PROC mode) | Port B MUX switches to host/ARM/GPU access |
| 3 | Write BRAM[6]=header_A, BRAM[7]=header_B | ARM input data written to shared BRAM |
| 4 | Write BRAM[0]=VecA, [1]=VecB, [2]=VecC | GPU BF16 input vectors written to BRAM |
| 5 | Write CMD=0x09 (PROC + arm_start) | ARM pipeline starts; BRAM[6,7] processed |
| 6 | Sleep 2s (or poll PROC_STATUS[0]) | ARM runs; adds 1 to each 32-bit field in BRAM[6..12] |
| 7 | Read BRAM[6], BRAM[7] | Verify ARM result (+1 per field) |
| 8 | Write CMD=0x11 (PROC + gpu_start) | GPU starts; fetches 6-instruction kernel from ROM |
| 9 | Poll PROC_STATUS[2] until gpu_done=1 | GPU executes: LD64x3 + BF_MAC + ST64 |
| 10 | Read BRAM[0] | Verify GPU result: 0x4060_4060_4060_4060 (3.5 x4) |

10. Synthesis Constraints (Xilinx ISE 10.1 / XST)

Both subgroups must follow these rules for correct synthesis on the Virtex-II Pro (xc2vp50, 23,616 Slices):

- No $readmemh: use case ROM for memory initialization

- Synchronous BRAM reads: always @(posedge clk) for reads; separate always blocks for write and read to allow BRAM inference

- BRAM inference hint: use (* RAM_STYLE = "BLOCK" *) on reg arrays to force BRAM instead of LUT RAM

- Pipeline_Reg flush: separate async reset (posedge rst) from sync flush (else if flush) — combined if(rst || flush) is not supported

- For-loop limit: XST 10.1 limits loop unrolling to 256 iterations; use sparse initial blocks instead

- No array indexing in always @(*): use case statements or flatten to individual registers

- XIL_TIMING_ALLOW_IMPOSSIBLE=1: required to bypass Pack:1653 when component delays alone exceed timing constraints; PAR still optimizes and meets timing in practice

11. Testing and Verification

11.1 GPU Unit Simulation (tb_gpu_top_wrapped)

Team 2 independently verified the GPU pipeline in ISim with four BF16 test cases: port B readback, BF_MAC 2.0*1.5+0.5=3.5, BF_MAC 1.0*1.0+0.0=1.0, BF_MAC 0.0*1.5+0.5=0.5. Result: PASS: 6 / FAIL: 0.

11.2 Joint Simulation (tb_lab9_joint)

A joint testbench connects a behavioral ARM model with the real GPU RTL and shared BRAM. Verified 8 stages including ARM +1 processing and GPU BF_MAC computation. Result: PASS: 10 / FAIL: 0.

11.3 Hardware Integration Test (lab9_joint_test_final.pl)

Final hardware test on NetFPGA board nf4.usc.edu:

nf_download nf2_top_par.bit && rkd & && perl lab9_joint_test_final.pl

| Test Case | Expected | Result |
| --- | --- | --- |
| ARM Header word 0 (+1) @ BRAM[6] | 00000002_00000003 | PASS |
| ARM Header word 1 (+1) @ BRAM[7] | 00000004_00000005 | PASS |
| GPU Vec A readback @ BRAM[0] | 40004000_40004000 | PASS |
| GPU Vec B readback @ BRAM[1] | 3FC03FC0_3FC03FC0 | PASS |
| GPU Vec C readback @ BRAM[2] | 3F003F00_3F003F00 | PASS |
| GPU BF_MAC result (3.5x4) | 40604060_40604060 | PASS |