EE 533 Lab 9 - Architecture Interface Specification

Network Processor with ARM CPU and GPU Tensor Core

Team 10 - Spring 2026

Shared Design Document for CPU Subgroup and GPU Subgroup

# 1. System Overview

This document defines the architecture and interface specifications for the integrated network processor system on NetFPGA. The system is developed by two subgroups: the CPU subgroup provides the ARM processor, Convertible FIFO, and system integration framework; the GPU subgroup provides the BFloat16 tensor processing unit. Both subgroups' modules connect to a shared Block RAM through a common interface defined in this document.

The system sits within the NetFPGA user_data_path module, replacing the standard passthrough stage. It captures network packets into shared BRAM, processes them with the ARM CPU and/or GPU, and sends modified packets back to the network.

# 2. Top-Level Architecture

## 2.1 Module Hierarchy

user_data_path.v

+-- fifo_top.v                    (system integration, registers)

+-- convertible_fifo.v         (shared BRAM, packet I/O)

|     Port A: network capture/send

|     Port B: MUX -> host / ARM / GPU

|

+-- arm_cpu_wrapper.v          [CPU SUBGROUP]

|     +-- arm_pipeline.v       (4-thread ARM pipeline)

|     +-- arm_alu.v            (32-bit ALU)

|     +-- arm_regfile.v        (64x32 register file)

|     +-- imem.v               (instruction ROM)

|     +-- dmem_32.v            (256x32 data memory)

|

+-- gpu_net.v                  [GPU SUBGROUP]

+-- bf16_adder_pipe.v    (BFloat16 adder)

+-- bf16_multiplier_pipe.v (BFloat16 multiplier)

## 2.2 Responsibility Boundary

| CPU Subgroup Provides | GPU Subgroup Provides |
| --- | --- |
| fifo_top.v (top-level wrapper) | gpu_net.v (tensor processor) |
| convertible_fifo.v (shared BRAM) | bf16_adder_pipe.v |
| arm_cpu_wrapper.v + ARM pipeline | bf16_multiplier_pipe.v |
| user_data_path.v (NetFPGA integration) | CUDA kernel + PTX translator |
| Register interface + control scripts | GPU test programs |
| Port B MUX logic |  |

# 3. GPU Module Interface (gpu_net.v)

This is the critical interface that both subgroups must agree on. The GPU module must implement exactly the following port list to plug into fifo_top.v:

## 3.1 Port Definition

| Direction | Signal | Width | Description |
| --- | --- | --- | --- |
| input | clk | 1 | System clock (125 MHz NetFPGA clock) |
| input | rst_n | 1 | Active-low reset (directly from ~reset) |
| input | start | 1 | Rising-edge pulse to begin GPU execution |
| output | done | 1 | High when execution complete (stays high until next start) |
| output | running | 1 | High while GPU is actively executing |
| output | bram_addr | 8 | BRAM address for read or write (0-255) |
| output | bram_wdata | 72 | Write data: {ctrl[7:0], data[63:0]} |
| input | bram_rdata | 72 | Read data: {ctrl[7:0], data[63:0]} (1-cycle latency) |
| output | bram_we | 1 | Write enable (active high, single-cycle pulse) |

## 3.2 Verilog Module Template

The GPU module must match this exact signature:

module gpu_net (

input  wire        clk,

input  wire        rst_n,

input  wire        start,

output reg         done,

output wire        running,

output reg  [7:0]  bram_addr,

output reg  [71:0] bram_wdata,

input  wire [71:0] bram_rdata,

output reg         bram_we

);

## 3.3 Timing Requirements

The BRAM has synchronous read with 1-cycle latency. The GPU must follow this protocol:

| Cycle | Action |
| --- | --- |
| T | GPU drives bram_addr = target address |
| T+1 | BRAM registers the address (internal latency) |
| T+2 | bram_rdata is valid and can be sampled |

For writes, the GPU drives bram_addr, bram_wdata, and bram_we=1 simultaneously. The write takes effect on the rising clock edge.

## 3.4 Control Protocol

The start signal is a single-cycle pulse generated by fifo_top when the host writes to the CMD register with bit[4]=1. The GPU must:

- On rising edge of start: assert running=1, deassert done, begin execution.

- During execution: running=1, done=0. Access BRAM freely via bram_addr/bram_we.

- On completion: deassert running=0, assert done=1.

- done must stay high until the next start pulse. Do NOT clear done in the idle state.

This is important because the host polls the PROC_STATUS register (0x2000124) to check done. If done is cleared immediately, the host may miss it.

# 4. Shared BRAM Memory Map

The Convertible FIFO BRAM is 256 entries x 72 bits. Each entry stores {ctrl[7:0], data[63:0]}. The BRAM is shared between the network FIFO, ARM CPU, and GPU. The following memory map defines which addresses each component uses:

| BRAM Address | Owner | Contents |
| --- | --- | --- |
| 0 | FIFO (network) | Packet word 0: IOQ module header (ctrl=0xFF) |
| 1 - 5 | FIFO (network) | Packet words 1-5: Ethernet + IP + ICMP headers |
| 6 | GPU input | vecA: 4 packed BFloat16 values (64-bit) |
| 7 | GPU input | vecB: 4 packed BFloat16 values (64-bit) |
| 8 | GPU output | bf16_add result: element-wise A + B |
| 9 | GPU output | bf16_mul result: element-wise A * B |
| 10 | GPU output | relu result: max(0, A) per lane |
| 11 | GPU output | dot product result: sum(A[i]*B[i]) in lane 0 |
| 12 - 13 | FIFO (network) | Remaining packet payload |
| 14 - 255 | Reserved | Available for future use |

## 4.1 BFloat16 Packing Convention

Each 64-bit data word contains 4 packed BFloat16 values in the following lane order:

data[63:48] = lane 3 (MSB)

data[47:32] = lane 2

data[31:16] = lane 1

data[15:0]  = lane 0 (LSB)

For dot product, the scalar result is placed in lane 0 (data[15:0]) with other lanes set to zero.

# 5. Port B Access Priority

BRAM Port B is shared between three consumers. The CPU subgroup implements a priority-based multiplexer in fifo_top.v:

| Priority | Consumer | Condition |
| --- | --- | --- |
| 1 (highest) | ARM CPU | arm_running == 1 (ARM is executing) |
| 2 | GPU | gpu_running == 1 (GPU is executing) |
| 3 (lowest) | Host (regwrite) | Neither ARM nor GPU is running |

This means the ARM and GPU must NOT run simultaneously. The host controls execution order via the CMD register. Typical sequence: run ARM first (data preparation), then run GPU (tensor computation).

# 6. Host Register Interface

The host controls the system through software-writable registers accessed via regwrite/regread commands. The GPU subgroup does not need to modify these registers; the CPU subgroup handles all register logic in fifo_top.v.

## 6.1 CMD Register (0x2000100) - Software Writable

| Bits | Field | Description |
| --- | --- | --- |
| [1:0] | mode | 00=IDLE (passthrough), 01=PROC, 10=FIFO_OUT, 11=FIFO_IN |
| [2] | reset | Write 1 to reset FIFO pointers |
| [3] | arm_start | Rising edge triggers ARM execution |
| [4] | gpu_start | Rising edge triggers GPU execution |

## 6.2 PROC_STATUS Register (0x2000124) - Hardware Readable

| Bits | Field | Description |
| --- | --- | --- |
| [0] | arm_done | 1 when ARM execution is complete |
| [1] | arm_running | 1 while ARM is executing |
| [2] | gpu_done | 1 when GPU execution is complete |
| [3] | gpu_running | 1 while GPU is executing |

# 7. System Execution Flow

The complete execution flow for a CUDA-mapped dot product operation:

- Host sets mode=FIFO_IN (0x03). Network packet arrives and is captured into BRAM[0..13].

- Host sets mode=PROC (0x01). Host can optionally write bf16 test vectors to BRAM[6] and BRAM[7] via regwrite.

- Host writes CMD=0x09 (PROC + arm_start). ARM processor runs: copies BRAM data to internal dmem, processes it, copies results back.

- Host polls PROC_STATUS until arm_done=1.

- Host writes CMD=0x11 (PROC + gpu_start). GPU reads BRAM[6] and BRAM[7], computes bf16 operations, writes results to BRAM[8..11].

- Host polls PROC_STATUS until gpu_done=1.

- Host reads results from BRAM[8..11] via regread, or sets mode=FIFO_OUT (0x02) to send the modified packet back to the network.

- Host sets mode=IDLE (0x00) to restore normal router passthrough.

# 8. Synthesis Constraints (ISE 10.1 / XST)

Both subgroups must follow these rules for the design to synthesize correctly on the Virtex-II Pro target:

- No initial blocks: use synchronous reset (rst or rst_n) for all register initialization.

- No $readmemh: use case ROM for instruction/data memory initialization.

- Synchronous reads for all memories: combinational reads (assign out = mem[addr]) cause XST to infer distributed RAM from flip-flops, which overflows the device. Use always @(posedge clk) for reads.

- No array indexing in always @(*): XST 10.1 cannot handle reg arrays indexed by variable in combinational blocks. Flatten arrays to individual registers or use case statements.

- No integer loop variables in for loops inside always blocks: use explicit unrolled assignments or genvar.

- Forward-declare all wires before use: Verilog in ISE 10.1 does not allow forward references.

# 9. Testing Procedure

## 9.1 Simulation (ISE ISim)

Each subgroup should independently simulate their module with a testbench that mimics the BRAM interface. The testbench should provide a simulated synchronous BRAM (array with clocked read/write) and verify all outputs against expected values.

## 9.2 FPGA Integration Test

After merging, the combined system is tested on NetFPGA using the fifoctl Perl script and the dot_product_test.pl script. The test writes known bf16 vectors to BRAM[6] and BRAM[7], runs the GPU, and reads back BRAM[11] to verify the dot product result.

## 9.3 CUDA Toolchain Test

The complete CUDA-to-hardware flow is verified by running ptx_to_gpu.py on the kernel.ptx output, which generates the expected register commands and verifies the bf16 arithmetic matches the hardware output.